43 inch outdoor vertical digital signage

Android version

Tech specification

Catalogue

Product Description	2

Display parameter	2

Touch parameters（Optional）	3

System configuration	3

System Function	3

Software function	4

Machine parameter	4

Electrical features	5

Packing list	5

Diagram	5

| version | drawing | audit |
| --- | --- | --- |
| 43-Android-13.0 |  |  |
| Revised date | 2024-07-29 |  |

Product Description

- Structural design: the shell is made of 1.0-2.5mm high-quality galvanized sheet, sprayed with outdoor special powder, and the outdoor anti-corrosion is more than 7 years.

- Waterproof design: the shell structure is designed with waterproof groove and waterproof labyrinth of air inlet and outlet holes, meeting the protection level of IP65.

- Dustproof design: The air inlet and outlet are designed with special dust-proof filter (medium and high efficiency dust-proof filter) to filter the fine dust in the air and prevent mosquitoes from entering the device.

- Anti riot glass: 6mm high permeability fully tempered glass is used as the protective tempered glass in front of the screen of the whole machine, with the light transmission coefficient ＞ 95%.

- Anti theft design: equipped with anti-theft door lock (padlock can be added, double anti-theft) to truly achieve anti-theft security. The fixing method is to fix the bottom expansion screw or embedded parts.

- High brightness, with air cooling system, and multi split screen display mode.

Display parameter

| Brand | QUNMAO |
| --- | --- |
| Model | QM-4318HW-L |
| Screen Size | 43inch |
| Resolution | 1920*1080/3840*2160（Optional） |
| Angle of view | 178°/178° |
| Contrast ratio | 3000：1 |
| Display size | 532(H) x 943(V)mm |
| Display rate | 16:9 |
| Back light | LED |
| Brightness | ≥2000cd/㎡ |
| Refresh rate | 60HZ |
| Color | 16.7M(8-bit) |
| Response time | 6ms |
| Life span | 50000h |
| Smart photosensitive | The intensity of outdoor light automatically adjusts the brightness of the LCD display screen |

- Touch parameters（Optional）

| Technology | Capacitive touch |
| --- | --- |
| Touch point | 20 points |
| Touch mode | Finger or touch pen |
| Touch accuracy | ≦2mm |
| Service life | 60 Million touch |
| - Drive mode | - Driver-free |
| - Camera | - 500MP |

System configuration

| Operation system | Android |
| --- | --- |
| Version | 13.0 |
| CPU | T972 Quad core A55，Main frequency up to 1.9GHz |
| GPU | Mali-G31 |
| RAM | 2GB |
| Storage memory | 16GB |
| Interface | HDMI*1(2.1 4kx2k@60Hz input) USB*2 |
| Audio interface | 3.5mm ear jack*1 |
| Network | 1pc，10M/100M Adaptive Ethernet |
| WIFI、BT | Built in WIFI（2.4G single-signal），BT4.2 |
| 3G/4G | Choose to install |

System Function

| Decoding Capability | Supports 4K video and picture decoding and playback |
| --- | --- |
| Software application | Support to install the third-party software |
| Language | Russian、English etc language |
| Boot Animation | Support to customize |
| Infrared remote control | Operate system by remote control |
| Audio function | Support left/right channel output,built-in double 8Ω/15W power amplifier |
| Power-on/off | Remote control/timing/network wake-up/self-start |
| System authorization | Allow root permission and customize software development |
| Mouse mode | support wired/wireless mouse or use keyboard |
| System update | Support local and remote upgrade |
| Play mode | Audio：MP3,WMA,WAV, APE, FLAC, AAC, OGG,M4A,3GPP etc |
|  | Picture：JPG、BMP、PNG etc |
|  | Video：AVI，rm，rmvb， MKV，WMV，MOV，MP4 etc |
| Typing | Standard Android keyboard as default,can install the third party typing |
| More functions | follow instruction manual/video or contact supplier |

Software function

| Background support | Web/mobile IE（local network/WAN） |
| --- | --- |
| Play mode | Networking/standalone |
| Play support | video、picture、subtitles、PPT、web、weather、clock etc mixture to play |
| Smart splicing | support custom playing modes to splice as setting |
| Timing function | support to set playing custom program in time period |
| Terminal administration | Administrator can create and modify account for level-to level administration |
| More function | follow instruction manual/video or contact supplier |

Machine parameter

| Color | black white |
| --- | --- |
| Body Material | Using 1.2-2.5mmSGCC high-quality galvanized steel plate |
| Protection grade | IP65 |
| Protective glass | 6mm explosion-proof glass (AG anti-glare/AR anti-ultraviolet optional) |
| Cooling by design | Intelligent direct ventilation (forced air cooling) heat dissipation system |
| Energy saving | Intelligent temperature control cooling system automatically adjusts wind speed and volume |
| dust proof | High-precision filtration system blocks 99% of dust particles |
| Product size | 650*1850*125mm(with base) |
| Base size | 650*500*60mm |
| Packing size | 1830*250*830mm |
| Bracket | Universal for horizontal/vertical hanging |
| Packing material | Carton |
| Net/gross weight | 70KG/71.5KG |
| Install ways | mount-on wall/embedding/hang/bracket |

Electrical features

| Scenarios | outdoor |
| --- | --- |
| Rated Voltage | AC100V-240V |
| Rated power | ≦100W-300W |
| Suitable Environment | -20℃ to +55℃ outdoor environment operation |
| Time switch | Support 16 groups of timed on/off, repeatable settings, timed on/off by day, week, month |
| Lightning protection | Level 3 |
| Leakage protection switch | Disconnect the circuit promptly when leakage occurs |

Packing List

| 43 inch outdoor vertical digital signage | 1pc |
| --- | --- |
| Power cable | 1pc |
| Remote control | 1pc |
| WiFi antenna | 1pc |
| Warranty card | 1pc |

Diagram